Supplementary Table 1. Protocol for horizontal polyacrylamide gel electrophoresis

| Reagents | For 4% Gel (mL) | For 5% Gel (mL) |
| --- | --- | --- |
| Deionized H2O | 12.2 | 11.6 |
| Acrylamide/bisacrylamide (30% 37.5:1) | 3.3 | 2.7 |
| Tris buffer (1.5 M, pH 6.0) | 5.0 | 5.0 |
| N,N,N′,N′-tetramethylethylene-diamine TEMED | 100 | 100 |
| Ammonium persulfate 10% | 35 | 35 |

The electrolyte chamber is filled with 3-times concentrated (0.6 M) tris(hydroxymethyl)aminomethane (Tris) buffer (3X Tris/glycine buffer), pH 6.0 (9.1 g/L Tris base and 43.3 g/L glycine in 800 mL deionized water, adjusted to pH 6.5 with HCl before making the final volume to 1 L). The comb and buffer were removed from the wells. Samples in sample buffer were loaded into the wells, and the gel was run at 30–45 V for 3 h in a cold room at 4 °C.