Supplementary File 2: TEG test

TEG (Haemonetics, TEG500, USA) is tested immediately after blood collection to avoid mesylate being metabolized by red blood cells. The preset temperature is 37℃. Place the detection cup on the instrument, and sequentially add 0.02 mL calcium chloride and 0.34 mL activated blood with a pipette. Operate according to the equipment manual, and record the TEG image. The results include reaction time (R), kinetics time (K), Angle, maximum amplitude (MA), coagulation index (CI#), and clot hardness (G), with the corresponding reference ranges: R = 5-10 m, K = 1-3 m, MA = 50 to 70 millimeters (mm), CI# = -3-3, and G = 4.5-11.0 d/sc.